We’ll build Project 1 slowly, and this time every step will answer three things:

- What are you doing

- Why are you doing it

- How to explain it in interview

At the end, we’ll package it cleanly for GitHub.

Project 1 goal

Build a small MES simulation:

Machine Simulator → MQTT Broker → MES Logic → Dashboard

What this project proves

By the end, you’ll be able to say:

I built a mini MES pipeline where a simulated machine publishes operational data over MQTT, a consumer layer calculates downtime and OEE metrics, and a dashboard visualizes real-time manufacturing performance.

The 5 parts of this project

We will rebuild it in this order:

- Mosquitto broker

- MQTT test messages

- Machine simulator in Node-RED

- MES logic in Node-RED

- Dashboard

- GitHub packaging

What you already have

You already installed:

- Mosquitto

- Node-RED

So we will start from there.

Phase 1 — Understand the broker

What is the broker?

Mosquitto is the middleman.

It does not create data.
It does not calculate OEE.
It only receives messages from publishers and sends them to subscribers.

Real factory meaning

- Machine or PLC publishes data

- MES subscribes to data

- Broker routes it

Interview line

Mosquitto acts as the MQTT broker, which decouples machine data producers from MES consumers.

Step 1

Open Command Prompt and run:

cd "C:\Program Files\Mosquitto"

mosquitto -v

What this does

- starts the broker

- -v means verbose logs

- keeps broker running in that window

What you should understand

When you see port 1883 listening, that means:

- broker is waiting for MQTT clients

- machine simulator and MES logic can now communicate through it

Phase 2 — Understand publish and subscribe

Before Node-RED, understand pure MQTT.

Step 2A — subscriber

Open a new terminal:

mosquitto_sub -h localhost -t test/topic

What this means

- mosquitto_sub = subscriber client

- -h localhost = connect to broker on your own laptop

- -t test/topic = listen to that topic

This terminal is now waiting for messages.

Step 2B — publisher

Open another terminal:

mosquitto_pub -h localhost -t test/topic -m "hello MES"

What this means

- mosquitto_pub = publisher client

- -m = message payload

What happened?

Publisher sent one message to broker.
Broker forwarded it to the subscriber listening on that topic.

Interview explanation

MQTT uses publish-subscribe communication. Producers publish to topics, and consumers subscribe to those topics through a broker, which removes direct point-to-point dependencies.

Phase 3 — Build a machine simulator

Now we simulate a machine because you don’t have a real PLC connected.

What the simulator should generate

Our fake machine will produce:

- state = RUNNING or STOPPED

- count = production count

- temp = machine temperature

Why these 3?

Because they are enough to mimic real MES signals:

- state helps calculate runtime/downtime

- count helps calculate output

- temp shows live process data

Real factory mapping

| Project signal | Factory meaning |
| --- | --- |
| state | machine status |
| count | produced units |
| temp | process parameter |

Step 3A — Start Node-RED

In terminal:

node-red

Open:

http://localhost:1880

What Node-RED is doing

Node-RED is your low-code runtime.
It lets you connect logic blocks called nodes.

Think of it like:

- PLC function blocks

- or visual ETL for real-time data

Step 3B — Import the simulator flow

Import this flow:

[

{

"id": "inject1",

"type": "inject",

"z": "flow1",

"name": "Every 2 sec",

"repeat": "2",

"once": true,

"topic": "",

"payload": "",

"payloadType": "date",

"x": 150,

"y": 100,

"wires": [["function1"]]

},

{

"id": "function1",

"type": "function",

"z": "flow1",

"name": "Machine Simulator",

"func": "let count = flow.get(\"count\") || 0;\nlet running = flow.get(\"running\");\n\nif (running === undefined) running = true;\n\nif (Math.random() < 0.2) running = !running;\n\nif (running) count += 5;\n\nlet temp = running ? (70 + Math.random()*10) : (60 + Math.random()*5);\n\nflow.set(\"count\", count);\nflow.set(\"running\", running);\n\nreturn [\n { topic: \"plant1/line1/machine01/state\", payload: running ? \"RUNNING\" : \"STOPPED\" },\n { topic: \"plant1/line1/machine01/count\", payload: count },\n { topic: \"plant1/line1/machine01/temp\", payload: Number(temp.toFixed(1)) }\n];",

"outputs": 3,

"x": 350,

"y": 100,

"wires": [["mqtt1"], ["mqtt2"], ["mqtt3"]]

},

{

"id": "mqtt1",

"type": "mqtt out",

"z": "flow1",

"name": "State",

"topic": "",

"qos": "1",

"retain": "true",

"broker": "broker1",

"x": 600,

"y": 60,

"wires": []

},

{

"id": "mqtt2",

"type": "mqtt out",

"z": "flow1",

"name": "Count",

"topic": "",

"qos": "1",

"retain": "false",

"broker": "broker1",

"x": 600,

"y": 100,

"wires": []

},

{

"id": "mqtt3",

"type": "mqtt out",

"z": "flow1",

"name": "Temp",

"topic": "",

"qos": "0",

"retain": "false",

"broker": "broker1",

"x": 600,

"y": 140,

"wires": []

},

{

"id": "broker1",

"type": "mqtt-broker",

"name": "Local Mosquitto",

"broker": "localhost",

"port": "1883"

}

]

Deploy it.

Understand the simulator code

This is the part you need to understand confidently.

1. Remember previous values

let count = flow.get("count") || 0;

let running = flow.get("running");

Meaning

- flow.get pulls stored values from memory

- if count doesn’t exist yet, start at 0

Why needed

Without memory, the count would reset every time the function runs.

Interview explanation

I used Node-RED flow context to maintain machine state across messages, such as production count and running status.

2. Initialize the machine

if (running === undefined) running = true;

Meaning

If this is the first cycle, start machine as running.

Why

Every simulator needs an initial state.

3. Randomly change state

if (Math.random() < 0.2) running = !running;

Meaning

- Math.random() gives a value between 0 and 1

- if it is less than 0.2, flip machine state

- 20% chance of change every cycle

Why

This simulates real plant behavior:

- machine runs

- machine stops

- machine restarts

Interview explanation

I introduced controlled random state transitions to simulate runtime and downtime events for OEE calculations.

4. Increment production count

if (running) count += 5;

Meaning

Only produce units when machine is running.

Why

Stopped machine should not produce count.

5. Simulate temperature

let temp = running ? (70 + Math.random()*10) : (60 + Math.random()*5);

Meaning

- if running, temp is higher

- if stopped, temp is lower

Why

This adds a realistic live process parameter.

6. Save state back to memory

flow.set("count", count);

flow.set("running", running);

Meaning

Store latest values for next cycle.

7. Return 3 messages

return [

{ topic: "plant1/line1/machine01/state", payload: running ? "RUNNING" : "STOPPED" },

{ topic: "plant1/line1/machine01/count", payload: count },

{ topic: "plant1/line1/machine01/temp", payload: Number(temp.toFixed(1)) }

];

Meaning

The function returns 3 separate messages:

- machine state

- machine count

- machine temp

Each goes to a different MQTT output.

Why the topics matter

These topics are structured like an industrial namespace:

plant1/line1/machine01/state

This means:

- plant1 = site

- line1 = line

- machine01 = machine

- state = variable

Interview explanation

I used a hierarchical MQTT topic design aligned to plant, line, machine, and signal, which makes subscriptions scalable and easier to govern.

Phase 4 — Verify machine messages

Open a terminal:

mosquitto_sub -h localhost -t plant1/line1/machine01/# -v

What this does

- # means subscribe to all subtopics under machine01

- -v shows topic and payload

Why this matters

You are verifying:

- simulator is publishing

- broker is routing

- topic naming is correct

Phase 5 — Build MES logic

Now we add the MES layer.

Its job is to:

- listen to machine topics

- track runtime and downtime

- calculate OEE metrics

Import this flow:

[

{

"id": "tab_mes_logic_1",

"type": "tab",

"label": "MES Logic",

"disabled": false,

"info": ""

},

{

"id": "mqtt_in_state_1",

"type": "mqtt in",

"z": "tab_mes_logic_1",

"name": "State In",

"topic": "plant1/line1/machine01/state",

"qos": "1",

"datatype": "auto",

"broker": "broker1",

"nl": false,

"rap": true,

"rh": 0,

"inputs": 0,

"x": 140,

"y": 100,

"wires": [["fn_mes_logic_1"]]

},

{

"id": "mqtt_in_count_1",

"type": "mqtt in",

"z": "tab_mes_logic_1",

"name": "Count In",

"topic": "plant1/line1/machine01/count",

"qos": "1",

"datatype": "auto",

"broker": "broker1",

"nl": false,

"rap": true,

"rh": 0,

"inputs": 0,

"x": 140,

"y": 160,

"wires": [["fn_mes_logic_1"]]

},

{

"id": "mqtt_in_temp_1",

"type": "mqtt in",

"z": "tab_mes_logic_1",

"name": "Temp In",

"topic": "plant1/line1/machine01/temp",

"qos": "0",

"datatype": "auto",

"broker": "broker1",

"nl": false,

"rap": true,

"rh": 0,

"inputs": 0,

"x": 140,

"y": 220,

"wires": [["fn_mes_logic_1"]]

},

{

"id": "fn_mes_logic_1",

"type": "function",

"z": "tab_mes_logic_1",

"name": "MES Logic",

"func": "let data = global.get('mesData') || {\n    state: 'UNKNOWN',\n    count: 0,\n    temp: 0,\n    downtimeSeconds: 0,\n    runtimeSeconds: 0,\n    goodCount: 0,\n    rejectCount: 0,\n    idealRunRatePerMin: 150,\n    startTs: Date.now(),\n    lastTs: Date.now(),\n    lastCount: 0\n};\n\nconst now = Date.now();\nlet elapsed = (now - data.lastTs) / 1000;\nif (elapsed < 0 || elapsed > 10) elapsed = 0;\ndata.lastTs = now;\n\nif (msg.topic.endsWith('/state')) {\n    data.state = String(msg.payload);\n}\nif (msg.topic.endsWith('/count')) {\n    data.count = Number(msg.payload);\n}\nif (msg.topic.endsWith('/temp')) {\n    data.temp = Number(msg.payload);\n}\n\nif (data.state === 'RUNNING') {\n    data.runtimeSeconds += elapsed;\n} else if (data.state === 'STOPPED') {\n    data.downtimeSeconds += elapsed;\n}\n\nconst producedDelta = Math.max(0, data.count - data.lastCount);\ndata.lastCount = data.count;\n\nif (producedDelta > 0) {\n    const rejects = Math.random() < 0.15 ? 1 : 0;\n    const good = Math.max(0, producedDelta - rejects);\n    data.goodCount += good;\n    data.rejectCount += rejects;\n}\n\nconst plannedTime = data.runtimeSeconds + data.downtimeSeconds;\nconst availability = plannedTime > 0 ? data.runtimeSeconds / plannedTime : 0;\nconst actualRatePerMin = data.runtimeSeconds > 0 ? (data.count / data.runtimeSeconds) * 60 : 0;\nconst performance = data.idealRunRatePerMin > 0 ? actualRatePerMin / data.idealRunRatePerMin : 0;\nconst totalProduced = data.goodCount + data.rejectCount;\nconst quality = totalProduced > 0 ? data.goodCount / totalProduced : 1;\nconst oee = availability * performance * quality;\n\nconst out = {\n    state: data.state,\n    count: data.count,\n    temp: data.temp,\n    runtimeSeconds: Math.floor(data.runtimeSeconds),\n    downtimeSeconds: Math.floor(data.downtimeSeconds),\n    goodCount: data.goodCount,\n    rejectCount: data.rejectCount,\n    availability: Number((availability * 100).toFixed(2)),\n    performance: Number((performance * 100).toFixed(2)),\n    quality: Number((quality * 100).toFixed(2)),\n    oee: Number((oee * 100).toFixed(2))\n};\n\nglobal.set('mesData', data);\nreturn { payload: out };",

"outputs": 1,

"x": 360,

"y": 160,

"wires": [["debug_mes_1"]]

},

{

"id": "debug_mes_1",

"type": "debug",

"z": "tab_mes_logic_1",

"name": "MES Output",

"active": true,

"tosidebar": true,

"console": false,

"tostatus": true,

"complete": "payload",

"targetType": "msg",

"x": 580,

"y": 160,

"wires": []

}

]

Deploy it.

Understand the MES logic

1. Load stored MES state

let data = global.get('mesData') || { ... };

Meaning

Pull prior MES values from global memory, or initialize them if first run.

Why global?

Because dashboard and MES logic are on different tabs.

Interview explanation

I used global context to share computed MES state across flows, especially between the calculation layer and dashboard layer.

2. Calculate elapsed time

const now = Date.now();

let elapsed = (now - data.lastTs) / 1000;

Meaning

Find how many seconds passed since last update.

Why

Runtime and downtime are time-based metrics.

3. Update current values

if (msg.topic.endsWith('/state')) { ... }

if (msg.topic.endsWith('/count')) { ... }

if (msg.topic.endsWith('/temp')) { ... }

Meaning

Use topic name to decide which field to update.

Why

A single function handles 3 incoming message types.

4. Runtime vs downtime

if (data.state === 'RUNNING') {

data.runtimeSeconds += elapsed;

} else if (data.state === 'STOPPED') {

data.downtimeSeconds += elapsed;

}

Meaning

- if running, add time to runtime

- if stopped, add time to downtime

Why

This is core MES logic.

Interview explanation

Availability was derived from cumulative runtime and downtime based on machine state transitions.

5. Production delta

const producedDelta = Math.max(0, data.count - data.lastCount);

data.lastCount = data.count;

Meaning

Figure out how much production increased since previous message.

Why

MES cares about increments, not just total count.

6. Good and reject counts

const rejects = Math.random() < 0.15 ? 1 : 0;

const good = Math.max(0, producedDelta - rejects);

Meaning

Simulate some scrap or reject rate.

Why

Quality calculation needs good vs reject.

7. OEE formulas

Availability

runtime / (runtime + downtime)

Performance

actualRate / idealRate

Quality

good / total

OEE

availability * performance * quality

Interview explanation

I implemented standard OEE logic where Availability reflects operating time, Performance compares actual throughput against the ideal run rate, and Quality measures good output against total production.

Phase 6 — Dashboard

Once your logic is working, the dashboard is only a display layer.

Its job is to show:

- state

- count

- temp

- downtime

- availability

- performance

- quality

- oee

If you want, in the next message I’ll give you a clean dashboard flow again, but this time I’ll explain each widget and each wire so you actually understand why it is there.

Phase 7 — GitHub structure

When the project is complete, your GitHub repo should look like this:

mes-mqtt-simulation/

├── README.md

├── flows.json

├── screenshots/

│   ├── simulator-flow.png

│   ├── mes-logic.png

│   └── dashboard.png

What goes into flows.json

Export all Node-RED flows.

What goes into README.md

- project summary

- architecture

- setup steps

- MQTT topics

- OEE formula

- screenshots

Best learning method from here

Don’t rush.

Do it in this exact way:

- rebuild simulator

- tell me in your own words what each line does

- rebuild MES logic

- tell me in your own words what OEE logic does

- then dashboard

- then GitHub

That will make you interview-ready.

Your immediate next step

Rebuild just these two parts now:

- broker running

- machine simulator flow imported and deployed

Then send me:

- whether the simulator is publishing, and

- your explanation of these 3 lines in your own words:

let count = flow.get("count") || 0;

if (Math.random() < 0.2) running = !running;

flow.set("count", count);

After that, I’ll teach you the MES logic the same way.